ST800 PLUS PPS v.1.1 default value of parameters and their allowed ranges

| # Panel | Frame | Field | # Default | Range |
| --- | --- | --- | --- | --- |
| Main | Pager Password | Current PW | None | 4 characters except special char |
|  |  | Replace check | Unchecked | N/A |
|  |  | Replace PW | None | 4 characters except special char |
|  | Address | Address 1 ~ 8 | 1 (Checked): 1234567 2 (Unchecked): Not Used 3 (Unchecked): Not Used 4 (Unchecked): Not Used 5 (Unchecked): Not Used 6 (Unchecked): Not Used 7 (Unchecked): Not Used 8 (Unchecked): Not Used | <Short address> ▷1 ~ 1,933,312 ▷1,998,849 ~ 2,031,614 <Long address> ▷2,101,249 ~ 3,223,326,720 ▷3,223,326,721~4,291,000,000 (Not recommended) |
|  | Addr Inc opt. | Address Increase option | Disable | Disable / Address 1 / The Others / All Addresses |
|  | - | Inventory Number | abc1234567 | 10 characters |
|  | Read / Write | Auto Write Enable | Unchecked | N/A |
|  |  | Interval | 5 sec | Number in second |
|  | PLL Frequency | Freq. | 929.9875 MHz | 929.0125 ~ 932.0000 MHz |
| Pager Options | Service | PLL IC | NPC SM5170AV | SAMSUNG KS8809 (Green PCB) / NPC SM5170AV (Blue PCB) |
|  |  | Collapse | 4 | 0 ~ 7 |
|  |  | Phase | Any Phase | Any Phase / Single A / Single B / Single C / Single D |
|  |  | Polarity | Normal | Normal / Inverse |
|  |  | Country | International | International / China |
|  | Display & Icon | Enable canned messages | Checked | N/A |
|  |  | Enable out of range check (Ant. Icon) | Checked | N/A |
|  |  | Display time | 16 sec | 8 / 16 / 32 / 64 sec |
|  |  | Show power up logo | Checked | N/A |
|  |  | Power Up Logo Edit | Unchecked | N/A |
|  |  | Power Up Logo | None | 12 characters consist of A(a)~Z(z), 0~9, ‘-‘. ‘[‘, ‘]’, ‘:’ or space. |
|  | Functions | Check duplicate message | Checked | N/A |
|  |  | GPS time setting | Unchecked | N/A |
|  |  | Enable auto on/off | Checked | N/A |
|  |  | Enable manner menu | Unchecked | N/A |
|  |  | Enable in range check | Unchecked | N/A |
|  | Alert | Low battery alert | Checked | N/A |
|  |  | Enable reminder alert | Checked | N/A |
|  |  | Enable end read alert | Unchecked | N/A |
|  | Source | Source display on | Checked | N/A |
|  |  | A2 (Address 2) | SOURCE2 | 11 characters consist of A(a)~Z(z), 0~9, ‘-‘. ‘[‘, ‘]’, ‘:’ or space. |
|  |  | A3 (Address 3) | SOURCE3 |  |
|  |  | A4 (Address 4) | SOURCE4 |  |
| Canned Messages | Canned Messages | Message 1 | HONEY | 12 characters consist of A(a)~Z(z), 0~9, ‘-‘. ‘[‘, ‘]’, ‘:’ or space. |
|  |  | Message 2 | FRIEND |  |
|  |  | Message 3 | LOVE YOU |  |
|  |  | Message 4 | HOME |  |
|  |  | Message 5 | OFFICE |  |
|  |  | Message 6 | CONGRATULATE |  |
|  |  | Message 7 | URGENT |  |
|  |  | Message 8 | MISS YOU |  |
| Configuration | Port | COM port | COM1 | COM1 ~ COM20 |
|  | Read Data Format | Text File Format (on Write Information Box) | Disabled | N/A |
|  |  | Window Screen Format (on PPS) | Enabled | N/A |
|  | Login Password | Current | None | ‘$’, ‘/’ and ‘:’ are prohibited. |
|  |  | New | None |  |
|  |  | Verify | None |  |

(2003-11-07)