Z2PLUS

# Pager Program Software v.2.0

User’s Manual

2003.8

* Contents

- Software explanation

- Installing software

  - Installing software

  - System requirements

- Starting/terminating software

  - Starting software

  - Terminating software

- Functions

  - Main panel

    - Address frame

    - PLL Frequency frame

    - Pager Password frame

    - Canned Message frame

    - Signal Settings frame

    - Power Up Screen frame

    - Serial Number frame

  - Option panel

    - Antenna Display frame

    - Date Setup frame

    - Time Display frame

    - Low Battery Alert frame

    - Sequential Lock Time frame

    - Out of Range Time frame

    - Character Type frame

  - Configuration panel

    - Parallel Port frame

    - Serial Port frame

    - Log In Password frame

    - Auto Write Interval frame

    - Read Data Format frame

  - Menu bar, Tool bar and Status bar

    - File menu

      - Open

      - Save

      - Print

      - Exit

    - Pager menu

      - Read

      - Write

      - Auto Write

      - Write Info

    - Setting menu

      - Configuration

      - Option

    - Help menu

      - About Z2PLUS PPS v.2.0

    - Tool bar

    - Status bar

- Default values of parameters and their allowed ranges

- Error messages and responses

- Hardware connection

  - Parallel Port

  - Serial Port

- Contact Info

- Software explanation

This software is to program Z2PLUS v.2.0 pager only. User must not program other Z2PLUS pager with this software. Also, Z2PLUS v.2.0 pager must be programmed by this v.2.0 PPS only. User must check the pager version and PPS version matching.

The software allows a user to program(write) addresses (Capcodes), PLL frequency, pager, canned messages, signal settings, power up screen, serial number, and various options(antenna display, date setup, etc.) to a pager.

The software has a function to let a user preview the write information before writing to a pager and print it out.  Also, it has the auto write function so that a user can write a great quantity of pagers automatically.  In addition to that, a user is able to read data from a pager and save the data to a file and load the file for future use.

The most upgraded feature on this version of software is to use both of parallel (LPT) and serial (COM) ports to communicate with a pager.  For parallel port (LPT) it allows a user to select either LPT1 or LPT2.  For serial port (COM) it has a range of choices from COM1 to COM20.

- Installing software

  - Installing software

Double click on setup.exe file, then Install Wizard starts.  Follow the instructions on Install Wizard to install software.  After installation a short-cut icon, ‘Z2PLUS PPS v.2.0’, is created on the desktop as well as under Start  Program.

  - System requirements

- Processor: Intel 486 or Pentium or better

- OS: Windows 95/98/2000/XP/ME, Windows NT 3.1 or later

- Hard disk space: Approximately 10 MBytes

- Monitor resolution: Best at 1024 x 768

- Starting/terminating software

  - Starting software

After successful installation of software Windows creates an icon for Z2PLUS PPS v.2.0 on the desktop.  Double clicking of the icon starts software.  Or, selecting Start  Program  Z2PLUS PPS v.2.0 starts software, too.  First window after starting software is Log In dialog box shown in <Figure 3.1>.  Entering proper password and clicking “OK” button allows to start the Pager Program Software (PPS).  Login password can be up to combination of 10 characters and numbers.  “Cancel” button terminates the Log In dialog box and PPS does not start.

<Figure 3.2> shows the main panel that appears right after clicking “OK” button on the Log In dialog box with the correct password.

<Figure 3.1> Log In dialog box

Menu bar

Tool bar

Status bar

<Figure 3.2> Main PPS screen

  - Terminating software

While software is running, “Alt + X”, little ‘x’ mark on the right top corner of software, or FileExit terminates software.  When terminating software <Figure 3.3> appears.

<Figure 3.3> Exit window

Clicking either “Yes” or “No” buttons terminate software.  However, “Yes” button saves the data, whereas “No” button does not.  “Cancel” button does not terminate software and go back to the main panel shown in <Figure 3.2>

- Functions

  - 4.1 Main panel

Main panel contains address (Capcode), PLL frequency, pager password, canned message, signal settings, power up screen and serial number.  <Figure 4.1> shows the main panel of software.

<Figure 4.1> Main panel

4.1.1 Address frame

Address frame can set total 3 addresses (Capcodes) and their enable/disable information.  Each Capcode can be either enabled or disabled.  The disabled address will not be programmed to pager. The range of Capcode is from 8 to 2097151. Note that the first Capcode field must be used to be able to write a pager.  Illegal Capcodes prompt an error message.

If you set the “Increase Mode” to “Disable”, then there will be no change to addresses even if you press the “Increase” or the “Decrease” buttons. Otherwise, if you set the “Increase Mode” to “Address 1 Increase” and press “Increase” or “Decrease”, then the value of Address 1 increases or decreases by one, respectively. Setting “All Increase” will affect on all addresses. While this “Increase Mode” is selected, after each writing procedure the corresponding addresses are increased by one automatically. However there will be no change to addresses if you select “Disable” mode, of course.

4.1.2 PLL Frequency frame

The allowed PLL frequency band is 100, 200, 400, 900MHz bands. Also frequency has to be multiple of the channel space. Illegal PLL frequency prompts an error message. However not all frequency in the band can receive signal. The receiving ability depends on RF board specification of the pager.

The Prescale value depends on the PLL chip model. PLL chip KS8809(SS) and SM5170(NPC). Usually Z2PLUS v.2.0 pager uses KS8809.

4.1.3 Pager Password frame

To be able to write or read to/from a pager, the correct pager password must be specified.  Pager password must consist of 4 characters or numbers. Current PW field must have the correct pager password to write or read to/from a pager.  If the pager password needs to be changed, then check on the “Replace”, then the edit box is activated to get the new pager password.  If you enter ‘/’, ‘:’ or ‘$’ characters or the length of input characters is not 4, then an error message prompts.

4.1.4 Canned Message frame

The number of canned message is 5, and each message can have up to 8 characters. It must not contain ‘$’, ‘/’ and ‘:’ characters. If so, then an error message is displayed.

For making a blank between two characters, press “Shift” and “Space bar” keyboard button together. Pressing “Space bar” button alone will run the program and write data to the pager.(See 4.4.2.2)

If you don’t check the “Use Canned Msg” check box, then no canned message will be programmed.

4.1.5 Signal Settings frame

“Data Polarity” can be either “Normal” or “Inverse”.  It is recommended not to change this value

unless you are sure to do so. (default: Normal)

“Channel Space” can be “6.25/3.125 KHz” or “5 KHz”. PLL Frequency of 100~199 can use “5 KHz” channel space, otherwise channel space is set to “6.25/3.125 KHz” automatically.

Pocsag protocol supports 512, 1200, 2400 bps of signal speed. Default value is 1200 bps.

4.1.6 Power Up Screen frame

It sets the LCD screen at pager when the pager is turned on.  It can have up to 12 characters. If you enter ‘/’, ‘:’, or ‘$’, then an error message prompts. (default: SUNTELECOM)

For making a blank between two characters, press “Shift” and “Space bar” keyboard button together. Pressing “Space bar” button alone will run the program and write data to the pager.(See 4.4.2.2)

If you don’t check the “Enable” check box, then no power up screen will be programmed.

4.1.7 Serial Number frame

Serial Number field cannot be edited.  It only displays the serial number of the pager that was read or written.  When writing a pager this serial number does not mean anything to a pager at all.

  - 4.2 Option panel

Option panel controls all the available options for Z2PLUS v.2.0 pager.  <Figure 4.2> shows the Option panel.  Option panel consists of Antenna Display frame, Date Setup frame, Time Display frame, Low Battery Alert frame, Sequential Lock Time frame, Out of Range Time frame and Character Type frame. To save the modification and exit click OK button.  Cancel button exits without saving the modification.  Each option is explained on the following subsections.

<Figure 4.2> Option panel

4.2.1 Antenna Display frame

When “In Range” is selected, antenna indicator symbol will disappear when the pager is too far away from paging system’s transmitter more than “Out of Range Time(see 4.2.6)”. If you select “Out of Range”, then antenna indicator symbol will always on the LCD.

4.2.2 Date Setup frame

This option programs the date of the pager. If you select “12:00A 1-1”, then the pager’s date and time will be programmed as it is. Otherwise if you select “Current”, then the current date and the time of the moment that programming the pager will be programmed to pager.

4.2.3 Time Display frame

This option programs the display of date and time.

4.2.4 Low Battery Alert frame

This option enables or disables the “Low Battery Alert” function of the pager.

4.2.5 Sequential Lock Time frame

This option programs sequential lock time. When the pager receives same message that already received in sequential lock time, the message is deleted automatically and displays symbol on LCD which indicates the pager has received the same message. The range of sequential lock time is 0~10 minutes. The “0” minute means that pager will not use this option.

4.2.6 Out of Range Time frame

This option programs out of range time. When the pager is out of pager system’s transmitting range for “Out of Range Time”, the antenna indicator symbol will disappear. (This option is meaningful when “Antenna Display” is only “In Range” mode.)

4.2.7 Character Type frame

This option programs the character type. Standard is used internationally. (Default : Standard)

  - 4.3 Configuration panel

Configuration panel controls the environment of software.  <Figure 4.3> shows the configuration panel. To save the modification and exit click OK button.  Cancel button exits without saving the modification.

<Figure 4.3> Configuration panel

4.3.1 Parallel Port frame

To use parallel port for the connection between a pager and a PC, select Parallel Port.  Then, LPT is enabled and it can be chosen between LPT1 and LPT2.  While parallel port is selected, “Check LPT Speed” button sets the fastest communication speed with a pager.  Clicking the ‘Check LPT Speed’ button prompts a message box shown in <Figure 4.4>.

<Figure 4.4> Check LPT Speed

Click “Yes” to proceed.  While checking the LPT speed a user must not change anything from software until one of following two message boxes shown in <Figure 4.5> and <Figure 4.6> is displayed.

<Figure 4.5> Fail to check LPT Speed

<Figure 4.6> Success to check LPT speed

4.3.2 Serial Port frame

Select Serial Port to use serial port to communicate with a pager.  To use serial port COM Port and Baud Rate must be set properly.  COM port can be selected from COM1 to COM20(But not all computers supports COM1 to COM20. Please check your computer’s COM port setting).  Baud Rate has the options of  4800, 9600, 19200 Since the current version of serial port buffer supports only Baud Rate of 9600, it must be set to 9600.  Serial port can be closed and opened manually with the “Open” and “Close” buttons.

If you want to make changes of port and baud rate, you have to close port first. Otherwise an error message will pop up.

4.3.3 Log In Password frame

To change the log in password, it must be done on this frame.  Enter the current password on Current PW edit box and the new password on New PW and Verify PW edit boxes.  They must not contain ‘:’, ‘/’ and ‘$’ characters.  New password and verify password must be the same.  Password can be up to 10 characters.  After writing passwords, click “Change” button to change the password.

4.3.4 Auto Write Interval frame

Auto Write Interval sets the interval value when Auto Write function is used.  For example if auto write interval is set to 10, then it writes a pager every 10 seconds.  Note that this value must be greater than the time duration of writing a pager.  For serial port the minimum value is 10; and for parallel port the minimum value is 3.

4.3.5 Read Data Format frame

This option decides how to display the read data from a pager when the Read command is executed.  If Text File Format is selected, then the read data is displayed in the info box shown in <Figure 4.7>. If Window Screen Format is selected, then the read data is applied to directly on UI screen.

<Figure 4.7> Information box for the read data from a pager

As shown in <Figure 4.7> there is a “PRINT” button.  Clicking this button makes a hard copy via a printer.  Click “CLOSE” button to close this window.

  - 4.4 Menu bar, Tool bar and Status bar

4.4.1 File Menu

File Menu consists of four sub-menus: Open, Save, Print and Exit.

4.4.1.1 Open

Open menu loads the file with the file extension of *.pgr.  Clicking “Open” menu prompts a Open File Dialog box shown in <Figure 4.8>.

<Figure 4.8> Open File Dialog box

4.4.1.2 Save

Save menu saves the data to a file with the file extension of *.pgr.  Clicking “Save” menu prompts a Save File Dialog box similar to <Figure 4.8>.  The saved file can be called by Open menu for future use.

4.4.1.3 Print

This menu makes a hard copy of data that is currently specified on the software.

4.4.1.4 Exit

Clicking Exit menu terminates software.  Before terminating software it prompts a window shown in <Figure 3.3>.

4.4.2 Pager Menu

Pager menu consists of four sub-menus: Read, Write, Auto Write and Write Info.

4.4.2.1 Read

Read menu reads the data from a pager.  This menu can be executed by pressing F5 key, also.  To read the data from a pager, the pager password must match with the password the pager holds.  The progress bar on the main panel shows how much of the reading has been done.  During the reading procedure ‘Reading..’ is displayed on the status bar of the main panel.  If the reading is successful, then ‘Success’ is displayed on the status bar of the main panel; whereas it displays ‘Failure’ and a separate error message window for failure of reading. Also software checks the read data whether the data is legal or illegal. If illegal data is in the read data, error message window will pop up. After successful reading, it displays the data according to the Read Data Format option from Configuration panel.

4.4.2.2 Write

Write menu writes the data to a pager.  This menu can be executed by pressing F6 key or space bar, also.  Before writing the data to a pager, software checks if there are any invalid values on writing data.  The progress bar on the main panel shows the how much of the writing has been done.  During the writing procedure ‘Writing..’ is displayed on the status bar of the main panel.  Like reading procedure, for successful writing it displays ‘Success’ on the status bar of the main panel; whereas it displays ‘Failure’ and a separate error message window for failure of writing.

4.4.2.3 Auto Write

Auto Write menu writes a pager automatically every specified interval on the Configuration panel.  This menu can be executed by pressing F7 key.  Clicking the menu one more time stops the auto writing procedure.  Also, disconnecting the pager from a PC stops the auto writing procedure.  While writing a pager the progress bar on the main panel moves and the status bar displays ‘Auto Writing’.  When it is done it displays ‘Success’ for successful writing or ‘Failure’ and a separate error message window for failure of writing.  Therefore, change to the next pager should be done when ‘Success’ is displayed on the status bar of the main panel.

4.4.2.4 Write Info

Write Info menu displays the data to be written to a pager on the info box shown in <Figure 4.9>.  As shown in the <Figure 4.9> there are two buttons: “PRINT” and “CLOSE”.  Clicking “PRINT” button makes a hard copy via a printer.  “CLOSE” button closes the box.

<Figure 4.9> Write Information box

4.4.3 Setting Menu

Setting menu consists of three sub-menus: Configuration, Option

4.4.3.1 Configuration

Configuration prompts Configuration panel shown in <Figure 4.3>.

4.4.3.2 Option

Option menu prompts Option panel shown in <Figure 4.2>.

4.4.4 Help

Help menu consists of only one sub-menu: About Z2PLUS PPS v.2.0

4.4.4.1 About Z2PLUS PPS v.2.0

About Z2PLUS PPS v.2.0 prompts the about box for software shown in <Figure 4.10>.

<Figure 4.10> About Z2PLUS PPS v.2.0

4.4.5 Tool bar

Tool bar has nine icons like shown in <Figure 4.11>.  From the left they are ‘Open’, ‘Save’, ‘Print’, ‘Read’, ‘Write’, ‘Write Info’, ‘Option’, ‘Configuration’ and ‘About’.  This tool bar allows a user to select one of those nine listed menus in quicker way.

<Figure 4.11> Tool bar

4.4.6 Status bar

Status bar displays many information.  It is located on the bottom of the main panel (see Figure 3.2).  Progress bar is located on the status bar.  Progress bar shows the how much of reading or writing from/to a pager is done so far.  There are two text panes on the right side of the progress bar.

The first one displays ‘Writing’, ‘Reading’, ‘Auto Writing’, ‘A.W. stopped’ , ‘Success’, and ‘Failure’.  ‘Writing’ is displayed during writing procedure, and ‘Reading’ for reading procedure. ‘Auto Writing’ is displayed during auto writing procedure, and ‘A.W. Stopped’ is displayed when  auto writing is stopped.  ‘Success’ and ‘Fail’ shows the result of writing or reading procedures.

The second one displays ‘COM’ when the COM port is selected from the Configuration panel.  Then, software checks the connection between the PC and serial port buffer every second.  If connection is made, then ‘Connect’ is displayed on the text pane whereas ‘Disconnect’ is displayed while there is no connection between them.  If LPT is selected from the Configuration panel, then ‘LPT’ is displayed on the text pane.

- Default value of parameters and their allowed ranges

| Panel | Fields | Default value | Range |
| --- | --- | --- | --- |
| Main | 1st Address/Enable | "1234567"/ Enable | 8~2097151 |
|  | 2nd Address/Enable | "0000000"/Disable | 8~2097151 |
|  | 3rd Address/Enable | "0000000"/Disable | 8~2097151 |
|  | PLL Frequency | 452.2500 MHz | 100~299,400~499,900~999 |
|  | PLL Chip | KS8809(SS) | KS8809(SS) or NPC(SM5170) |
|  | Pager Password | "1234" (BCD Format) | 4 characters or numbers |
|  | Use Canned Msg | Checked | Check or Uncheck |
|  | 1st Canned Message | HONEY | 8 character or numbers |
|  | 2nd Canned Message | FRIEND | 8 character or numbers |
|  | 3rd Canned Message | LOVE YOU | 8 character or numbers |
|  | 4th Canned Message | HOME | 8 character or numbers |
|  | 5th Canned Message | OFFICE | 8 character or numbers |
|  | Data Polarity | Normal | Normal or Inverse |
|  | Channel Space | 6.25/3.125 KHz | 6.25/3.125 KHz or 5 KHz |
|  | Signal Speed | 1200bps | 512, 1200, 2400 bps |
|  | Enable Power Up Screen | Checked | Check or Uncheck |
|  | Power Up Screen | SUNTELECOM | Under 12 characters or numbers |
|  | Serial Number | 0000000000 | Under 10digit of numbers |
| Option | Antenna display | In range | In range or Out of Range |
|  | Date Setup | 12:00A 1-1 | 12:00A 1-1 or Current |
|  | Time Display | 12:00A 1-1 | 12:00A 1-1 or 1-1 12:00A |
|  | Low Battery Alert | Enable | Enable or Disable |
|  | Sequential Lock Time | 0 minute(Disable) | 0~10 |
|  | Out of Range Time | 5 minutes | 1~30 |
|  | Character type | Normal | Normal, China I, China II |
| Configuration | Parallel Port Enable | Disabled | Enable or Disable |
|  | LPT Number | LPT1 | LPT1 , LPT2 |
|  | Serial Port Enable | Enabled | Enable or Disable |
|  | COM Port | COM1 | COM1~COM20 |
|  | Baud Rate | 9600 bps | 4800, 9600, 19200 bps |
|  | Auto Write Interval | 15 sec | Serial : 10~99 sec |
|  |  |  | Parallel : 3~99 sec |
|  | Read Data Format | Window Screen Format | Text File Format or Window Screen Format |

- Error Messages and responses

| Error Number | Contents | Reason and Responses |
| --- | --- | --- |
| 101 | Incorrect Login password. Try again | Reason: Incorrect login password is given Response: Enter the correct login password (case-sensitive) |
| 102 | $', '/' and ':' are illegal characters for Log in password | Reason: Illegal characters are entered where prohibited. Response: Delete illegal characters. |
| 103 | New and Verify passwords do not match | Reason: New and verify passwords are different. Response: Make sure they are the same. |
| 105 | Pager password must consist of 4 numbers | Reason: Pager password is not given, or it is not 4 characters. Response: Give the 4-character pager password. |
| 106 | New Pager password must consist of 4 numbers | Reason: New pager password is not given, or it is not 4 characters. Response: Give the 4-character new pager password. |
| 107 | Wrong pager password! Or, bad connection! | Reason: Wrong pager password or bad connection. Response: Check connection and give the correct pager password. |
| 108 | EEPROM is empty! Or, bad connection! | Reason: Empty EEPROM or bad connection. Response: Check connection and if the connection has no problem, then bring the pager to the factory. Empty EEPROM can only be programmed in Powermode. |
| 109 | Bad connection or Trying to write the empty EEPROM in User mode. | Reason: Empty EEPROM or bad connection. Response: Check connection and if the connection has no problem, then bring the pager to the factory. Empty EEPROM can only be programmed in Powermode. |
| 110 | Bad connection or Trying to read the empty EEPROM in User mode. | Reason: Empty EEPROM or bad connection. Response: Check connection and if the connection has no problem, then bring the pager to the factory. Empty EEPROM can only be programmed in Powermode. |
| 112 | First Capcode must be used! | Reason: Writing to a pager with empty first Capcode. Response: Use the first Capcode. |
| 113 | Illegal Capcode(s) in Address | Reason: Illegal Capcode is entered or read from pager. Response: Check the Capcode range. |
| 114 | Illegal Capcode Enable flag | Reason: Illegal Capcode enble flag is entered or read from pager. Response: Check the Capcode enable flag. |
| 115 | Illegal PLL Frequency | Reason: Illegal PLL frequency is entered or read from pager. Response: Check the PLL frequency |
| 116 | Illegal PRESCALE value | Reason: Illegal Prescale value is entered. Response: Check the Prescale value. |
| 117 | Illegal PLL Frequency, Frequency is not multiple of channel space | Reason: Illegal PLL frequency is entered or read from pager. Response: Check the PLL frequency |
| 118 | $', '/' and ':' are illegal characters for Canned Message | Reason: Illegal characters are entered where prohibited. Response: Delete illegal characters. |
| 119 | Illegal Data Polarity | Reason: Illegal Data Polarity value is entered. Response: Check the Data Polarity value. |
| 120 | Illegal Channel Space | Reason: Illegal Channel space is entered or read from pager. Response: Check the Channel space value. |
| 121 | Illegal Signal speed | Reason: Illegal Signal speed is entered or read from pager. Response: Check the Signal speed value. |
| 121_1 | Illegal Character type | Reason: Illegal character type is entered or read from pager. Response: Check the character type value. |
| 122 | Power up screen have to be characters or numbers | Reason: Illegal characters are entered where prohibited. Response: Delete illegal characters. |
| 124 | Option Not Initialized | Reason: Options are not initialized.. Response: Make sure all options are entered correctly.. |
| 125 | Illegal BS Setup Time | Reason: Illegal Battery saving time was entered or read from pager. Response: Check the Battery saving time values. |
| 126 | Illegal Out of Range Time | Reason: Illegal Out of range time was entered or read from pager. Response: Check the Out of range time values. |
| 127 | Illegal Sequential Lock Time | Reason: Illegal Sequential lock time was entered or read from pager. Response: Check the Sequential lock time values. |
| 134 | Illegal Chip ID | Reason: Illegal Chip was used in pager. Response: Check the Pager CPU’s model name. |
| 135 | Minimum auto write value for parallel port is 3 seconds! Minimum auto write value for serial port is 10 seconds! | Reason: Entered the number less than minimum allowed Auto writing interval value. Response: Change the Autowrite interval value. |
| 136 | Illegal LPT Port Number | Reason: Entered illegal LPT Port number. Response: Check the LPT port number. |
| 137 | Illegal Communication Channel | Reason: Communication channel(LPT or COM) is not selected. Response: Check the communication channel is selected. |
| 138 | Illegal Read Data Format | Reason : Read data format is not selected. Response: Check the read data format is selected. |
| 139 | Loading info file failed! Make sure the file is for Z2PLUS PPS v.2.0. | Reason : .pgr file or info.ini file is corrupted. Response: Don’t open .pgr files and delete info.ini file. If you run the program, info.ini file will be created with default values automatically. |
| 140 | Loading Config file failed! Make sure the file is for Z2PLUS PPS v.2.0. | Reason : config.ini file is corrupted. Response: Delete config.ini file. If you run the program, config.ini file will be created with default values automatically. |
| 141 | Loading Map file failed! Make sure the file is for Z2PLUS PPS v.2.0. | Reason : Z2PLUS20.map file is corrupted. Response: Delete Z2PLUS20.map file. If you program the pager, Z2PLUS20.map file will be created with default values automatically. |
| 142 | Making Config file failed | Reason : Illegal values were entered for config parameters. Response: Check the entered config parameter values. |
| 143 | Error during initializing the parallel port | Reason: Error for using parallel port. Response: Terminate software and run again. |
| 144 | Failure during reading procedure via LPT | Reason: Error during reading procedure via LPT. Response: Check the connection and try again. Or, do the Check LPT Speed on Configuration panel. |
| 145 | Failure during writing procedure via LPT | Reason: Error during writing procedure via LPT. Response: Check the connection and try again. Or, do the Check LPT Speed on Configuration panel. |
| 146 | Error while creating ThreadWatchComm | Reason: Error for using serial port. Response: Terminate software and start again. |
| 147 | Queue is full | Reason: Error for using serial port. Response: Terminate software and start again. |
| 148 | Fail to initialize COM port! | Reason: Error for using serial port. Response: Open COM port manually from Configuration panel. |
| 149 | Port is already opened! | Reason: Trying to open another COM port while one of the COM port is opened. Response: If you want to open other port, close the COM port first, and open it. |
| 150 | Serial Port and BaudRate are not initialized | Reason: COM Port number and Baudrate is not initialized or illegally selected. Response: Close the COM port first, select the correct COM port and Baudrate. |
| 151 | Close Port before make change COM port number or Baudrate | Reason: Trying to change the COM port or Baudrate while COM port is opened.. Response: Close the COM port first, and open it. |
| 152 | Write Command Nak during serial comm. | Reason: Error for using serial port buffer Response: Check the connection with serial port buffer. Try again after the connection is confirmed. |
| 153 | Write Data Nak during serial comm. | Reason: Error for using serial port buffer Response: Check the connection with serial port buffer. Try again after the connection is confirmed. |
| 154 | Read Command Nak during serial comm. | Reason: Error for using serial port buffer Response: Check the connection with serial port buffer. Try again after the connection is confirmed. |
| 155 | Read Data was not received well during serial comm. | Reason: Error for using serial port buffer Response: Check the connection with serial port buffer. Try again after the connection is confirmed. |
| 156 | COM Check Connection NAK before Read Procedure | Reason: Error for using serial port buffer Response: Check the connection with serial port buffer. Try again after the connection is confirmed. |
| 157 | COM Check Connection NAK before Write Procedure | Reason: Error for using serial port buffer Response: Check the connection with serial port buffer. Try again after the connection is confirmed. |
| 158 | COM Hardware Information NAK before Read Procedure | Reason: Error for using serial port buffer Response: Check the connection with serial port buffer. () Try again after the connection is confirmed. |
| 159 | COM Hardware Information NAK before Write Procedure | Reason: Error for using serial port buffer Response: Check the connection with serial port buffer. Try again after the connection is confirmed. |
| 160 | Tx and Rx Does not Match! Please try again. | Reason: Error for using serial port buffer Response: Check the connection with serial port buffer. Try again after the connection is confirmed. |

7. Hardware connection

7.1 Parallel Port

PC

Pager

To COM port

To pager

Serial Port buffer

Power

jig

PC

Pager

To LPT1 or LPT2

To pager

jig

ParallelBuffer

7.2 Serial Port

- Contact Info

## Company

HANTEL CO., LTD.

#513-15, Suntecity, Sangdaewon-Dong,

Jungwon-Gu, Seongnam, Kyunggi-Do

462-806, South Korea

TEL: 82-31-737-3568

FAX: 82-31-777-2330

#### Developer

##### Kwon, Dong-won

TEL: 82-31-737-3568

Cell Phone: 82-11-668-9101

E-Mail: xirtam@han-tel.com